- Table of Contents

1	Introduction	6

1.1	Design Philosophy	6

1.1.1	Environmental and Electrical Survivability	6

1.1.2	Robust Power Distribution	6

1.1.3	Mission Resilience and Safety	7

1.1.4	Electromagnetic Compatibility (EMC)	7

1.1.5	Serviceability and Field Upgradeability	7

1.2	Functional Domains.	8

1.2.1	Power Supply	8

1.2.2	Communication Interfaces	8

1.2.3	Sensors	8

1.2.4	USER INTERFACE SUBSYSTEM	9

1.2.5	Processing	9

2	Power Supply Subsystem	10

2.1	Power Supply Protection and Filtering	10

2.1.1	Overview and Design Criteria	10

2.1.2	Reverse Polarity Protection	11

2.1.3	ESD and Load Dump Protection	12

2.1.4	Over-voltage & Current-limiting	13

2.1.5	Analysis and Failure Mode	14

2.1.6	EMC	16

2.2	Power Supplies	18

2.2.1	12 V Power Domain	18

2.2.2	Primary Power Domain (5.0 Volt)	19

2.2.3	Digital Logic Domain (3.3 Volt)	20

2.2.4	Analog Sensor Domain (8.0 Volt)	21

3	Communications SUBSYSTEM	24

3.1	CANBUS Interface	24

3.1.1	CANBUS Signal Conditioning	24

3.2	Power Supply	24

3.2.1	TTL I/O	25

3.2.2	Slope Control	25

3.2.3	Signal Integrity Simulation	25

3.3	Legacy Serial Interface	25

3.3.1	Interface Conditioning and Power	26

3.3.2	SeaTalk I Operation (Single Wire, RX/TX)	26

3.3.3	3.2.4 NMEA 0183 Operation (RX Only)	27

3.3.4	Physical Connector	27

4	Sensors	29

4.1	Power Sensor	29

4.1.1	Voltage Sensing	29

4.1.2	Current Sensing	29

4.2	Temperature Sensor	30

4.2.1	Internal Microcontroller Temperature Sensor	30

4.2.2	External Display Temperature Sensor	30

4.3	Ambient Light Sensor	30

4.3.1	OPT3004 Digital Ambient Light Sensor	31

4.3.2	Firmware Use	31

4.4	Wind Transducer Interface	31

4.4.1	Circuit DESIGN	31

4.4.2	Compatible Wind Transducers	33

5	USER Interface	36

5.1	Overview and HMI Elements	36

5.2	Power Supply	36

5.3	Touch Display	36

5.3.1	Characteristics	36

5.3.2	Communication	37

5.4	Touch Screen Operation	37

5.4.1	Timeout	37

5.4.2	Wet-Weather Usability	37

5.4.3	Grounding Considerations on Metal-Hulled Vessels	38

5.4.4	Further Testing	38

5.5	Audio Alerts	39

6	Expansion Connectors	40

6.1	ESP-PROG	40

6.1.1	Flash Programmer	40

6.1.2	JTAG Debugging Port	40

6.2	I2C	40

6.3	One-Wire	40

7	Processor	41

7.1	Description	41

7.2	flash storage partitioning	41

7.3	GPIO Mapping	41

7.4	WiFi	41

7.5	Bluetooth/BLE	41

8	PCB Layout and Thermal Design	42

8.1	Board Stackup	42

8.2	Earth Planes	42

8.3	Functional Layout	42

8.4	Thermal Design	42

8.5	Mechanical	42

8.5.1	Connectors	42

8.5.2	Dimensions	42

8.5.3	Fasteners	42

8.5.4	Fabrication strategy	42

9	7. Validation and Test Plan	43

10	8. Bill of Materials Summary	44

11	9. Conclusion and Future Work	45

12	References	46

- Quick reference

GPIO Allocation

# Introduction

The MDD400 is a compact data display unit designed for integration with modern marine and recreational vehicle (RV) electrical systems. It is optimised for the NMEA 2000/CANBUS backbone, displaying data via a serial HMI interface driven over UART. Offloading graphical updates to the display controller allows the onboard microcontroller to focus on interfacing, protocol translation, and data processing, supporting a wide range of sensor and bus configurations.

## Design Philosophy

The MDD400 was developed for deployment in marine environments, where electrical systems are exposed to harsh physical conditions and frequent voltage disturbances. However, recognising the similarity between marine and automotive/RV 12 V systems — particularly in terms of wiring topology, fault conditions, and user expectations — the MDD400 has been designed with potential future certification for road-going vehicles in mind.

The overarching philosophy is to design for realistic abuse scenarios, reducing the likelihood of damage in the field and thereby eliminating the need for return-to-base repairs. This strategy supports the planned 24-month, no-questions-asked replacement warranty — a competitive differentiator intended to rival established marine brands while avoiding the overhead of setting up global repair facilities.

This section outlines the design principles, performance goals and safety considerations that inform the entire hardware design.

### Environmental and Electrical Survivability

The MDD400 must survive common transients found on 12 V DC supply rails, including:

load dump conditions up to 150 V (per ISO 7637-2 Pulse 5b);

reverse polarity connection of 12–14 V;

electrostatic discharge (ESD), following IEC 61000-4-2;

conducted and radiated emissions (per CISPR 25 / EN 55025);

and ground bounce and other anomalies from long DC wiring typical in boats and RVs.

To ensure compatibility with both marine and automotive domains, protection circuitry has been tested against the most demanding load dump waveforms from ISO 7637-2, and both power supply and communication interfaces have been isolated or filtered to maintain signal integrity and protect sensitive components.

### Robust Power Distribution

Four separate power rails are implemented:

primary power domain, supplying a 5 V rail at up to 2.0A peak using a switching regulator;

digital logic domain, supplying  a regulated low-ripple 3.3 V rail using a linear LDO regulator;

analog sensor domain, supplying a 8V rail at up to 100mA using a linear LDO for the wind sensor interface.

Each includes current limiting, thermal protection, and either undervoltage lockout (UVLO) or controlled enable logic. Combined with a discrete surge stopper circuit, the system is designed to self-recover from faults without the use of fuses or mechanical resets.

The 5 V rail is generated from the 12 V input using a high-efficiency synchronous buck converter, reducing power loss while providing a stable intermediate supply. This 5 V rail feeds the serial display and acts as the input to the 3.3 V LDO, ensuring low ripple on the digital logic rail. This architecture improves efficiency over directly stepping 12–14.8 V down to 3.3 V, while retaining the noise performance and simplicity of LDO regulation where it matters most.

The 8 V rail is implemented using a low-dropout linear regulator and is used exclusively for the masthead analog wind transducer. The transducer draws less than 50 mA under normal operation, making the linear approach thermally acceptable and electrically clean. The regulator is selectively enabled via [WIND_EN], conserving power when the wind interface is not in use.

### Mission Resilience and Safety

Critical circuits — such as power monitoring, communication interfaces and display — remain functional under abnormal voltage conditions. The system does not rely on any software-controlled start-up sequence or central controller to protect hardware against electrical faults. Where possible, faults are detected and isolated at the analog level.

Additionally:

the device is protected against short-circuits and sustained over-voltage;

no fuse is required to isolate faults — the MOSFET protection circuit self-recovers;

dual-input polarity protection ensures that legacy serial and NMEA2000 inputs remain independent.

### Electromagnetic Compatibility (EMC)

EMC compliance was a foundational design requirement. Particular attention was given to:

separation of analog and digital ground planes;

use of common-mode chokes, pi filters and LC filtering at all external interfaces;

low-inductance return paths and surface-mount TVS diodes;

and shielding/isolation of NMEA2000 and legacy serial connectors.

The product is designed to meet both CE and FCC Class B emissions limits and includes on-board suppression to meet ISO 11452-2 and ISO 7637-2.

### Serviceability and Field Upgradeability

Firmware updates and configuration are supported over Bluetooth/BLE. In production, firmware flashing is performed using a pogo-pin programming jig that interfaces with a 6-pin 0.5 mm pitch header footprint compatible with the ESP-PROG programmer. The 6-pin header, while fitted to prototypes, is not populated in the final assembly. It may be included in a development or open-source variant. Currently, the programming interface includes polarity protection for power pins but lacks further ESD or transient protection — this will be reviewed in future revisions.

The internal regulators and protection circuits are designed to handle most common wiring faults gracefully, allowing field servicing with minimal tools or expertise.

The design is prepared for dual certification paths: marine installations via NMEA2000 guidelines and CE/FCC emissions certification, with provisions for future expansion into the recreational vehicle (RV) space.

## Functional Domains.

Key hardware features include:

CANBUS interface for communicating with the NMEA 2000 backbone;

TFT LCD display with capacitive touch screen connected via high-speed UART;

ambient light sensor connected to the I²C bus;

temperature and power sensing systems for protection and diagnostics;

robust, fault-tolerant hardware and interfaces designed for marine electrical environments;

optional plug-and-play legacy serial data interface (e.g. NMEA 0183 or SeaTalk-compatible) on selected models;

support for analog wind transducers (angle and speed) on selected models;

Bluetooth/BLE for configuration and firmware updates via mobile app; and

Wi-Fi-ready for future protocols such as SignalK and OneNet.

The hardware is organized into five functional domains (subsystems), as shown in the block diagram:

### Power Supply

The system is powered from a 12 V source via either the NMEA 2000 network or the (optional) legacy serial connection. Input power passes through reverse polarity protection, surge and current limiting, and an EMI filter before being regulated to:

5 V for display and logic circuits (via SMPS);

3.3 V for the digital domain (via LDO); and

8 V for the analog wind transducer interface (via LDO) on selected models.

All three regulators are designed to balance cost, robustness, and efficiency in line with the MDD400’s overall mission to survive real-world abuse without requiring repair or user intervention.

### Communication Interfaces

NMEA 2000 (CANBUS) via the ESP32-S3’s internal TWAI peripheral and external CAN transceiver; and, on selected models

a RS422 compatible, plug&play interface that accepts legacy marine serial protocols such as NMEA 0183 and SeaTalk®.

### Sensors

The following sensors are used by the MDD400:

ESP32-S3 integrated temperature sensor for diagnostics and protection;

analog current and voltage sensors for self-monitoring and diagnostics;

Texas Instruments OPT3004 ambient light sensor on the I²C bus for automatic display dimming; and

wind transducer inputs for analog wind angle and frequency-based wind speed.

### USER INTERFACE SUBSYSTEM

A custom DWIN DMG48480F040_01WTC[15] capacitive touchscreen display is used as user interface. The 4” TFT LCD display has a resolution of 480×480 pixels and communicates with the ESP32-S3 over UART using the DGUS II protocol.

### Processing

An Espressif ESP32-S3 microcontroller module handles all system logic, sensing, and communication. It manages protocol parsing, sensor readings, and updates to the display. The module includes Bluetooth/BLE for configuration and firmware updates, with Wi-Fi reserved for future features.

# Power Supply Subsystem

## Power Supply Protection and Filtering

### Overview and Design Criteria

The electrical environment aboard small vessels shares many characteristics with automotive systems, but with greater variation and typically less standardisation. Marine electrical systems may incorporate multiple battery banks — often with separate cranking and house systems — and are increasingly adopting lithium chemistries such as LiFePO₄. It is not uncommon for vessels to include 24 V or 48 V subsystems or to operate some loads from inverter-generated AC power.

Despite this variability, the communication and sensor networks relevant to the MDD400 are standardised to operate from nominal 12 V supplies. The NMEA 2000 backbone, as well as legacy protocols such as SeaTalk and NMEA 0183, are all 12 V-based. These systems are generally unregulated, powered by user-installed cabling, and often exposed to transients caused by inductive loads, battery switching, or alternator events. The MDD400 must tolerate these conditions while maintaining safe operation of downstream circuitry.

Designing for this environment requires careful attention to both transient suppression and steady-state fault protection. The MDD400 input protection circuit is modelled on best practices from automotive design, particularly those outlined in ISO 7637-2[1]. While this standard is not mandatory in marine applications, it provides a useful baseline for evaluating and simulating real-world transient events.

The MDD400’s power input protection strategy is defined by the design criteria in Table 1, reflecting expected conditions on small vessel 12 V systems. These criteria guided the selection and simulation of each protection stage.

Table 1: Power Supply Protection Design Criteria

| Protection Function | Design Criteria |
| --- | --- |
| Reverse Polarity | Survive continuous reverse connection of ±12 V; no damage; automatic recovery |
| Load Dump and Surge Clamping | Survive ISO 7637-2 Pulse 5b (150 V, 400ms exponential decay); limit to < 60 V |
| ESD Protection | Tolerate ±15 kV air discharge per IEC 61000-4-2[2] |
| Over-voltage Limiting | Disconnect load above 18.5 V; reconnect below 18.5 V without latch-up |
| Current Limiting | Limit current to ~1.0 A; tolerate sustained overloads without damage |
| EMC Filtering | Suppress conducted emissions above 1 MHz; limit conducted noise to < 100 mV p-p; contain radiated emissions to meet FCC Part 15 and EN 55032 Class B limits |

These functions are implemented discretely to reduce cost, improve component availability, and allow field observability. Where appropriate, the design includes thermal protection and is engineered to fail safe under fault conditions.

The sections that follow describe each function in detail.

### Reverse Polarity Protection

Both 12 V inputs — NMEA 2000 and legacy serial — are protected from reverse polarity using discrete Schottky diodes.

A MOSFET-based reverse protection scheme was considered but not adopted. The primary justification for using a simple diode is the presence of generous headroom in the input voltage range (nominal 12 V vs. 5 V and 3.3 V regulated rails), which makes the forward voltage drop acceptable. Diode protection offers simpler implementation, lower component cost, and greater resilience to fault modes such as latch-up or shoot-through.

The primary component is the SS34 (DO-214AC/SMA package) [3], which provides reliable protection against reverse connections while introducing minimal voltage drop in normal operation:

Maximum Reverse Voltage (VRRM): 40V

Average Forward Current (IF(AV)): 3.0A

Surge Current Rating (IFSM, 8.3ms): 100 A

Typical Forward Voltage Drop (VF @ 1 A): ~0.5V

Reverse Leakage Current: ~0.5 mA at 40V

This approach ensures automatic recovery from incorrect wiring and protects downstream circuitry by blocking reverse current. The forward voltage drop is acceptable given the headroom available between the 12 V input and the downstream regulators.

A solder jumper in parallel with the diode is included on the legacy serial interface for development and testing. This allows the interface to be used as a 12 V power source for external equipment if required, while maintaining galvanic isolation from the NMEA 2000 backbone.

This approach ensures automatic recovery from incorrect wiring and avoids damage to sensitive downstream components. Under reverse polarity conditions, the Schottky diode blocks current flow entirely, and no reverse current is conducted.

A solder jumper in parallel with the diode is included on the legacy serial interface for development and testing. This allows the interface to be used as a 12 V power source for external equipment if required, while still maintaining galvanic separation from the NMEA 2000 backbone.2.1.3 Load Dump and ESD

Describes the expected surge conditions (including alternator disconnect), ESD threats, and the use of a high-power TVS diode to clamp spikes safely.

### ESD and Load Dump Protection

The first line of defence against both electrostatic discharge (ESD) and high-energy surge events is a high-power transient voltage suppressor (TVS) diode. The SM8S36CA [16] is used at the 12 V input, upstream of the surge stopper MOSFET, to clamp and absorb energy during overvoltage events.

This diode begins clamping at approximately 58 V and is rated for 6.6 kW peak pulse power (10/1000 µs waveform). It is bidirectional, allowing it to suppress both positive and negative transients relative to system ground. During a surge event — such as alternator load dump, battery disconnection under load, or inductive spike — the SM8S36CA diverts energy away from sensitive downstream circuits by rapidly entering avalanche breakdown.

This component was selected specifically for its compatibility with ISO 7637-2 Pulse 5b[1] waveforms, including worst-case unsuppressed load dumps of up to 150 V. Simulation and test confirm that the clamped voltage at the downstream surge stopper FET remains within safe limits during these conditions.

The TVS diode also contributes to ESD protection, supplementing local filtering and layout strategies. It is capable of absorbing ±30 kV contact discharges when tested per IEC 61000-4-2 [2], with negligible leakage under normal operating voltages.

Its placement upstream of the MOSFET allows it to protect the input circuitry without exposing internal logic or regulation components to surge currents.

Simulation of the SM8S36CA response to an ISO 7637-2 Pulse 5b event — approximated as a 150 V exponential decay with a time constant of 80 ms — confirms that the diode clamps the downstream voltage to a maximum of 58 V throughout the transient. This provides over 40% margin relative to the 100 V absolute maximum of the surge stopper MOSFET and 65 V rating of the primary SMPS controller [16]. The diode’s power absorption capability exceeds the energy of the test waveform, ensuring robust protection even during worst-case alternator disconnect or overvoltage conditions.

The simulated energy absorbed by the SM8S36CA during a worst-case ISO Pulse 5b event is approximately 16.7 J, well within the capabilities of this device. With a peak pulse power rating of 6600 W (10/1000 µs waveform), the diode offers ample headroom for marine surge events. This safety margin ensures long-term reliability even under repeated transient exposure.

### Over-voltage & Current-limiting

The over-voltage and current-limiting functionality is implemented using a discrete surge stopper circuit built around a P-channel MOSFET, two bipolar junction transistors (BJTs), and a high-side shunt resistor.

The over-voltage cutoff is defined by a resistive voltage divider connected to the input rail. When the divided voltage exceeds the base-emitter threshold of a monitoring PNP transistor (approximately 0.6–0.7 V), the transistor begins conducting. This pulls the gate of the P-channel MOSFET upward, switching it off and disconnecting the load. The resistor values are selected to yield a trip point of approximately 18.5 V, sufficient to protect all downstream regulators and components from accidental overvoltage conditions — for example, an alternator operating without a battery load.

The current-limiting function is based on a 0.68 Ω high-side shunt resistor. A second BJT monitors the voltage across the shunt. When the drop exceeds ~0.7 V (corresponding to ~1.0 A), this transistor activates and also pulls up the MOSFET gate, disabling the load path. This mechanism protects the regulator and filter stages from sustained overcurrent events such as short circuits or excessive inrush.

The MMBT5401 PNP transistors used to sense current and voltage and drive the MOSFET gate have a Vce of 150V and a maximum Ice of 600 mA, improving reliability under high dV/dt switching conditions and providing a faster turn-off transition for the MOSFET. This ensures that both over-voltage and over-current protections activate swiftly and effectively.

The operation of this circuit has been validated under load dump simulation and current injection tests. Transients above the trip point result in sharp MOSFET cutoff with minimal overshoot. The system automatically recovers when the fault condition clears, ensuring seamless protection without requiring external intervention.

A simulated ISO 7637-2 Pulse 5b transient (150 V peak, 80 ms decay) was applied to the input. When the input exceeds the 18.5 V trip threshold, the gate voltage begins rising rapidly through the 4.7 Ω gate resistor and 100 nF equivalent capacitance. Once the gate voltage crosses the MOSFET’s Vgs(off) threshold (approximately –2 V), the device switches off, and the output is suppressed to near zero. The simulated response confirms that the MOSFET turns off within microseconds of the over-voltage condition, limiting downstream exposure and avoiding stress to the 42 V-rated linear regulator and other sensitive circuitry.

The circuit also incorporates hysteresis, which prevents the MOSFET from oscillating on and off near the trip point. When the MOSFET switches off due to an over-voltage event, the load is disconnected and the voltage at the upper leg of the voltage divider rises slightly, maintaining the transistor in conduction. The MOSFET only re-enables once the input voltage falls well below the trip threshold, ensuring stable operation during slow-falling or noisy input conditions.

### Analysis and Failure Mode

Each of the three onboard power supply rails — the 5 V SMPS, 3.3 V LDO, and 8 V LDO — incorporate integrated protection features to ensure safe and reliable operation. These include short-circuit protection, thermal shutdown, and undervoltage lockout (UVLO), as detailed in sections 2.2, 2.3 and 2.4 respectively.

This section summarises the simulated and expected performance of each input protection element under worst-case load dump conditions. It also identifies their operating margins and expected failure modes.

Reverse Polarity Protection

The SS34 Schottky diode used on each power input is rated for 3.0 A average forward current and 100 A surge for 8.3 ms. In reverse polarity scenarios, the diode blocks current flow with minimal leakage (~0.5 mA at 40 V). The diode recovers automatically when correct polarity is restored. Based on its electrical characteristics and the low impedance of marine supply wiring, this component can safely tolerate sustained reverse connection of 12–14 V without damage or thermal stress. No failure mode is expected within the defined design limits.

The SS34 Schottky diode used on each power input provides low-loss polarity protection with a typical forward voltage drop of approximately 0.5 V. The diode is rated for 3.0 A continuous forward current and can tolerate surge currents up to 100 A for 8.3 ms. To verify its suitability for transient load dump conditions, a time-series simulation was performed using the worst-case ISO 7637-2 Pulse 5b profile.

In this scenario, the diode was modelled conducting up to 88 A peak current with a decaying profile over approximately 43 ms. The simulated energy dissipation was approximately 2.66 J, resulting in a calculated peak junction temperature of ~146 °C assuming a thermal capacitance of 0.45 J/°C and 40 °C ambient. This remains below the device’s 175 °C absolute maximum junction temperature.

The results confirm that the SS34 operates within thermal and electrical ratings even during extreme surge events. While the diode is operating close to its limit, the simulated performance under these rare transients justifies its use in this design. The part recovers automatically once correct polarity is restored and no failure mode is expected under normal or reverse connection scenarios.

Transient Suppression (TVS Diode)

The SM8S36CA is placed upstream of the MOSFET and begins clamping at 36 V, reaching a maximum clamped voltage of approximately 58 V during surge conditions. Simulation of ISO 7637-2 Pulse 5b (150 V peak, 80 ms exponential decay) using time-series integration shows the diode absorbs approximately 19.3 J of energy. Assuming a conservative thermal capacitance of 0.16 J/°C and ambient temperature of 40 °C, the peak junction temperature is calculated at approximately 148 °C — safely below the device’s 175 °C absolute maximum rating.

The expected temperature of the PCB pad beneath the TVS is estimated at ~145 °C during this event, based on typical thermal gradients and copper pour area. This remains below solder softening thresholds for SAC305, with no long-term degradation expected. The TVS has sufficient energy capacity to withstand multiple such transients without failure.

Surge Stopper Circuit (MOSFET and BJT Controller)

The over-voltage protection circuit operates effectively up to the ~18.5 V threshold, disconnecting the load using a fast-switching P-channel MOSFET. Simulation confirms that downstream voltage rises only marginally before the MOSFET turns off within a few microseconds. Overshoot is negligible, and sensitive components (e.g., the 42 V-rated LP2951 LDO) remain protected.

The MMBT5401 PNP transistors used to detect over-voltage and over-current events and drive the MOSFET gate are rated for 150 V collector-emitter voltage and 600 mA collector current. These ratings provide ample margin for the application and ensure fast and reliable MOSFET turn-off during surge events.

The MOSFET does not conduct during the surge peak due to early turn-off, and hence does not dissipate significant power during the load dump. The calculated MOSFET temperature remains close to ambient.

Failure modes would typically require either:

a surge above ~180 V sustained for >100 ms, exceeding the SM8S36CA energy capacity;

extended clamping that pushes junction temperature above 175 °C; or

a fault disabling the over-voltage detection transistor, delaying MOSFET shutoff.

None of these conditions are expected in the defined operating environment, and the design includes sufficient margin to survive realistic automotive or marine transients.

The combined protection system performs robustly during surge and fault conditions, ensuring continued safe operation of the downstream regulation and logic circuitry.

Sensitivity Analysis – Surge Voltage Tolerance

To evaluate the robustness of the protection scheme under more severe conditions, time-series simulations were performed for ISO 7637-2 load dump pulses with peak voltages of 150 V, 175 V, 200 V, 225 V, and 250 V, each decaying exponentially over 400 ms. Both the TVS (SM8S36CA) and the Schottky diode (SS34) were evaluated. While both were found to operate within their respective SOA at or below 175 V, simulations show that above this level the TVS junction temperature exceeds 175 °C and cumulative energy approaches the device’s failure threshold. The results confirm the selected components are suitable for ISO 7637-2 Pulse 5b (up to 150 V), but operation beyond 175 V is not guaranteed without risk of failure.

Short-Circuit Protection (Current Limiter)

To evaluate the behaviour of the surge stopper circuit under a downstream short-circuit condition, a simulation was performed assuming the load is abruptly shorted to ground while the input remains at 13.5 V. The current through the MOSFET initially spikes to approximately 10 A, but is rapidly clamped by the over-current detection circuit. This spike lasts for less than 5 µs before the current stabilises at approximately 0.96 A — the threshold set by the 0.68 Ω current sense resistor and the V_BE turn-on voltage of the PNP sense transistor.

During the initial 5 µs, the MOSFET momentarily dissipates up to 67 W. Thereafter, steady-state dissipation is ~13 W, and junction temperature stabilises at 107 °C assuming continuous operation and 40 °C ambient. This confirms that the MOSFET (IRFR5410TR, rated for 40 W) operates safely under permanent short-circuit conditions. No thermal runaway or SOA violation is observed.

The current-limiting function of the protection circuit provides robust defence against hard short-circuits on the output. No fuses are required. The circuit self-recovers when the fault is cleared, ensuring high availability and fault resilience in the target environment.

### EMC

The MDD400 is designed to meet both conducted and radiated electromagnetic emissions requirements for CE (EN 55032 Class B[17]) and FCC Part 15 compliance, while ensuring robustness against electromagnetic immunity threats in line with ISO 11452-2[18] and ISO 7637-2[18]. To this end, a combination of discrete filtering components and PCB layout strategies are employed to suppress emissions and protect sensitive circuitry.

#### Conducted Emissions

The primary 12 V input is filtered using a multi-stage EMI suppression network consisting of:

a π-filter topology with 22 µF ceramic input capacitors, 10 nF mid-filter capacitor, and additional 22 µF output capacitors;

a high-performance shielded common-mode choke (SMCM7D60-132T) rated for 1.3 A;

a 100m Ω sense resistor for diagnostic current monitoring; and

a 600 Ω @ 100 MHz ferrite bead (FB3) to attenuate high-frequency common-mode noise to/from the switching regulator domain.

This network effectively attenuates conducted emissions above 1 MHz and isolates switch-mode regulator noise from reaching the vessel's 12 V supply lines. The design target was to suppress switching harmonics to < 100 mV peak-to-peak, as observed at the 12 V input under full load.

#### Radiated Emissions

Careful segregation of analog, digital, and high-current switching grounds helps reduce loop areas and suppress radiated emissions. Key measures include:

low-inductance return paths using filled copper pours;

edge-stitched ground planes isolating external connector regions;

strategic placement of high-frequency capacitors near connector pins (e.g., 100 pF across CAN_H/L); and

use of differential layout for CAN and legacy serial lines.

#### CANBUS Interface Filtering

The CAN interface is isolated and filtered using:

15 pF capacitors from each CAN line to chassis ground (NET-C);

a 100 pF differential capacitor between CAN_H and CAN_L;

a high-isolation common-mode choke (ACT45B-510-2P-TL003);

an NUP2105L TVS array for ESD and transient suppression.

This filtering approach is consistent with the recommendations of ISO 11898-2[19] and helps prevent both emissions and susceptibility to EMI propagating through the NMEA 2000 network.

#### Legacy Serial Port Filtering

The optional 12 V legacy serial interface is also filtered using:

100 µH inductors on power and signal lines;

100 pF shunt capacitors for high-frequency filtering; and

a SMF15CA TVS diode for signal line ESD and surge protection.

Combined with layout isolation from the CAN domain, this ensures that emissions from external devices do not couple into the main PCB circuitry.

#### Grounding and Isolation Strategy

The MDD400 employs separate digital (GND) and connector/chassis (NET-C) ground domains. These are joined only at carefully controlled locations, typically through the shield of common-mode chokes or designated net-ties. This strategy minimizes ground bounce, avoids ground loops, and enhances immunity to conducted and radiated transients.

By implementing these design strategies, the MDD400 is able to comply with applicable EMC regulations while maintaining reliable operation in electrically noisy marine and automotive environments.

#### Wireless Subsystem (Wi-Fi and Bluetooth)

The MDD400 incorporates the Espressif ESP32-S3 microcontroller, which includes integrated Wi-Fi and Bluetooth/BLE radios. The module carries CE and FCC modular certification, having been tested to comply with FCC Part 15 Subparts C and E, EN 300 328 and EN 301 489 under the Radio Equipment Directive (RED)[16]. Provided the integration guidelines (antenna layout, decoupling, trace clearance) are followed — as they are in the MDD400 — no further radiated emissions testing is required at the system level.

## Power Supplies

### 12 V Power Domain

The MDD400 is supplied from a nominal 12 V input, which serves as the master power domain for the entire system. This 12 V rail is filtered, protected against load-dump and ESD events, and regulated into three downstream voltage domains:

5 V SMPS: used primarily for the LCD backlight and as the intermediate supply for the 3.3 V LDO;

3.3 V LDO: supplies the ESP32 MCU and other digital circuitry;

8 V LDO: powers the optional analog wind transducer.

Protection features include:

An over-voltage cutoff at ~18.5 V, implemented by a discrete surge stopper circuit;

Input current limiting at ~960 mA during short-circuit or overload conditions.

Power consumption was assessed under three scenarios:

Day – daytime use with the display backlight at 100% and wireless transmitters (Wi-Fi and Bluetooth) off;

Night – nighttime use with display backlight reduced to 1% and wireless transmitters off;

Peak – full daytime use with display backlight at 100% and Wi-Fi/Bluetooth active.

Table 2.4 summarises the typical and peak 12 V domain power consumption across these three operating conditions.

Table 2.4 Power Summary across Domains

| Domain | Night | Day | Peak |
| --- | --- | --- | --- |
| 5 V SMPS (LCD only) | ~92 mA | ~245mA | ~245 mA |
| 3.3 V LDO (digital domain) | ~329 mA | ~328mA | ~542 mA |
| 8 V LDO (only with wind transducer) | ~16 mA | ~16 mA | ~18 mA |
| Total current @ 12 V | ~208mA | ~277 mA | ~376 mA |
| Power | ~2.5 W | ~3.3 W | ~4.5 W |

*Backlight current at 1% brightness estimated to be ~20 mA, based on display datasheet and linear PWM scaling.

Based on these estimates, the MDD400 draws ~208 mA under night-time use and up to ~317 mA under peak conditions. This corresponds to a Load Equivalency Number (LEN) of 2–7, well within the NMEA 2000 maximum permitted value of LEN = 20 [20].

The RV-C specification does not impose explicit power consumption limits or standardized power budgeting mechanisms akin to the NMEA2000 specification’s LEN, so there's no inherent protocol-level enforcement to prevent excessive power draw by individual devices. System designers and integrators bear the responsibility of ensuring that the cumulative power consumption of RV-C devices does not exceed the capabilities of the RV's power distribution system.

In practice, this necessitates careful planning and consideration during the design and integration phases. Manufacturers and installers must assess the power requirements of each RV-C device and ensure that the total load remains within safe and functional limits. This approach helps maintain system stability and prevents potential issues arising from power overconsumption on the network.

If you're integrating devices like the MDD400 into an RV-C network, it's advisable to:

review the power consumption specifications of each device.

ensure that the RV's power distribution system can handle the cumulative load.

consult the RV-C Layer document for any updates or recommendations regarding power management.

The design ensures compatibility with marine and RV networks and prioritises energy efficiency—an important consideration for solar-powered or battery-limited systems such as sailboats.

### Primary Power Domain (5.0 Volt)

The 5 V rail is derived from the filtered 12 V input using a buck-mode switching regulator (TPS54560B-Q1[6]). It provides up to 5 A peak current and serves as the intermediate supply for the 3.3 V LDO as well as directly powering the serial LCD display. The regulator offers:

wide input range up to 60 V;

integrated high-side MOSFET;

thermal shutdown and current limit protection; and

adjustable soft-start and switching frequency.

The switching frequency is set to 1.25 MHz. The output is filtered using a 22 µH power inductor and a 47 µF low-ESR ceramic capacitor, with additional bulk capacitance as required. A 600 Ω @ 100 MHz ferrite bead (FB3) on the output further suppresses high-frequency noise.

The SMPS layout closely follows Texas Instruments’ guidelines, with a compact switching loop, tight input/output capacitor placement, and careful separation of power and analog grounds. The SMPS section is isolated by local copper pours, connected to the global ground plane through a high-frequency ferrite.

Display power is gated by a P-channel MOSFET controlled via [DISP_EN]. This allows firmware to shut down the 5 V rail to the display, reducing standby power or resetting the DGUS display controller if required.

### Digital Logic Domain (3.3 Volt)

The 3.3 V digital logic domain is powered by an AMS1117-3.3 [7] low-dropout (LDO) linear regulator, using the 5 V rail as its input. This domain powers the ESP32-S3 microcontroller, CAN transceiver, I²C sensors, and analog signal conditioning stages.

Capacitive filtering on the LDO output includes:

10 µF local bulk capacitance;

100 nF decoupling capacitors placed near all major loads; and

five 10 µF + 100 nF RC pairs at the ESP32, CAN transceiver, and op-amp stages.

The AMS1117 features internal current limiting and thermal shutdown. Under typical operating conditions, the 3.3 V rail draws approximately 405 mA, with short-term peaks up to 640 mA. These values are distributed across the subsystems as shown in Table 2.

Thermal dissipation is mitigated through extensive copper pours on all four PCB layers, all tied to the LDO output pin (Pin 2 / Tab). A 3×3 via array connects the top and bottom layers, and a soldermask opening on the bottom layer improves convective cooling. This configuration ensures safe junction temperatures under all expected load conditions. Under peak load conditions of ~640 mA, with a 5 V input and 3.3 V output, the AMS1117 dissipates approximately 1.1 W. Based on a conservative thermal resistance estimate of ~30 °C/W with good copper heatsinking, the resulting temperature rise is under 35 °C. Given a 40 °C ambient environment, the regulator operates well below its 125 °C junction temperature limit. These results confirm that the AMS1117 is adequately specified for both average and peak operating conditions in the MDD400.

Table 2: Digital Domain (3.3v) Loads

| Load Component | Typical Current | Peak Current |
| --- | --- | --- |
| ESP32-S3 | 250 mA | 450 mA* |
| Ambient light sensor (OPT3004) | 0.2 mA | 0.2 mA |
| Dual op-amps (TLV9002IDR ×2) | 2.6 mA | 4 mA |
| Comparator (LM393DR) | 1 mA | 1.5 mA |
| CAN transceiver (SN65HVD234) | 74 mA | 85 mA |
| Pull-up loads | ~1 mA | ~1 mA |
| Total | ~329 mA | ~552 mA |

*Peak current estimate reflects ESP32-S3 operation with active Wi-Fi transmission at high data rates, based on Espressif datasheet specifications.

### Analog Sensor Domain (8.0 Volt)

#### The 8 V supply is provided using a linear LDO (LP2951-50DR[22]), supplied from the unregulated, protected 12 V rail. The regulator can be disabled if no wind transducer is connected.

To protect this output against miswiring (such as direct short to ground), a current-limiting circuit is implemented using discrete components, followed by a voltage monitor that informs the microcontroller of fault conditions.

#### Load

The optional 8 V analog rail is dedicated to powering legacy masthead wind transducers, including popular transducers from Raymarine /Autohelm and Navico (B&G, Simrad).  We studied several generations of these transducers and found them to present very similar electrical loads. We briefly discuss the Raymarine  E22078/9 and B&G 213 transducers below as a typical examples.

Raymarine  E22078/9 - The primary load for Raymarine sensors is a Melexis Sentron 2SA-10 dual-axis Hall-effect sensor, which draws a steady supply current of approximately 16 mA as per its datasheet [26]. In addition, the transducer includes two TL914 dual op-amps that buffer the X/Y vane signals and provide local power conditioning for the sensor. While these op-amps and associated passive components contribute additional quiescent current, the total consumption of the wind transducer is estimated to remain below 25 mA in normal operation. The anemometer circuit, which uses an opto-interrupter with a series 560 Ω resistor, contributes transient pulses but negligible average current. Combined, the entire wind transducer system is well within the 100 mA output rating of the LP2951-50 LDO.

B&G 213 Masthead Unit – The B&G 213 transducer operates nominally at 6.5 V and has been verified to tolerate input voltages up to 8 V. Diagnostic documentation indicates the use of analog vane signals with phase-shifted sinusoidal outputs (Red, Green, Blue) and a pulse-type wind speed output on the Violet line. Under normal operation, the B&G 213 unit draws significantly less than 100 mA, with total current estimated around 25–30 mA, consistent with its analog signal architecture and passive sensing elements. Compatibility testing confirmed stable operation of the unit at 8.0 V, with no signal distortion or thermal issues observed.

#### Design

The LP2951 uses a resistor divider on the feedback pin (pins 6 and 7) to adjust the output to 8.0 V. It features internal current limiting, thermal shutdown, and a precision reference. The shutdown pin (pin 3) is controlled via [WIND_EN], allowing the 8 V rail to be disabled in firmware when the wind sensor is not in use. The regulator input is protected using a 10 Ω series resistor (R68) to limit inrush current and short-circuit stress and a 600 Ω @ 100 MHz ferrite bead (FB7) is placed on the input to attenuate high-frequency conducted noise.

The output includes a low-leakage BAT54 Schottky diode (D1) to prevent reverse current flow, and an additional ferrite bead (FB1) to improve EMI suppression at the analog interface connector. Output filtering comprises a 10 µF bulk capacitor and a 100 pF high-frequency bypass capacitor, along with a feedback RC network (R17/R18 and C48) for loop stability.

The LP2951’s dropout voltage of less than 0.6 V at 100 mA ensures reliable regulation even under low input conditions, with wide tolerance to input transients thanks to the upstream protection network.

#### Analysis

Thermal analysis shows that, under maximum expected load of 50 mA, power dissipation is approximately 0.24 W (for a 4.8 V drop at 50 mA), resulting in a temperature rise of less than 15 °C on the PCB copper pour. The regulator remains well within its 125 °C junction temperature rating, even in high ambient conditions.

#### Current Limiting

The 8 V output is protected by a discrete linear current-limiting circuit. This design uses a P-channel MOSFET as a high-side pass element, driven by an NPN transistor that monitors the load current via a 12 Ω shunt resistor. When the output current exceeds 50 mA, the circuit enters a controlled current-limiting mode to protect both the device and external wiring from overheating or damage due to miswiring or short circuits.

In normal operation, the gate of the P-MOSFET is pulled down to ground through a 470 Ω resistor. With the source at 8 V and the gate at 0 V, the gate-source voltage is –8 V, ensuring the MOSFET is fully enhanced and offers minimal resistance to the load. A 7.5 V Zener diode between gate and source limits the maximum gate-source voltage to a safe level during power-up and limiting conditions.

Current flowing to the load develops a voltage across the 12 Ω shunt resistor. When the load current exceeds approximately 50 mA, the resulting voltage (~0.6 V) turns on the NPN transistor, which begins pulling the gate of the MOSFET upward via a 2.2 kΩ resistor. As the gate voltage rises toward the source, the MOSFET's conduction decreases, causing it to enter its linear region and limit the current supplied to the load. This analog feedback loop maintains the output current near the limiting threshold until the fault is removed.

A small 470 pF capacitor is connected in parallel with the sense resistor to speed up the response of the current limiter to fast transients, such as capacitive inrush or sudden short circuits. This ensures the transistor turns on promptly and the MOSFET begins throttling before significant energy is delivered to the fault. The discrete design avoids the need for sacrificial or resettable fuses, and integrates smoothly with the downstream voltage-monitoring comparator that informs the microcontroller of persistent faults.

#### Undervoltage Monitoring

The 8 V power output to the wind transducer is monitored by a comparator-based circuit that allows the microcontroller to detect fault conditions such as output undervoltage, short circuits, or regulator dropout. This enables firmware to take appropriate recovery actions and provide user feedback in the event of wiring errors or external faults.

The monitoring circuit is built around a low-power comparator with open-drain output. The inverting input of the comparator is connected directly to the 8 V output through a 12 kΩ resistor and is filtered by a 1 nF capacitor to ground. This configuration allows the inverting input to closely track the output voltage while rejecting high-frequency noise.

The non-inverting input receives a fixed reference voltage derived from the 12 V protected supply rail via a resistive divider consisting of 22 kΩ and 10 kΩ resistors. This divider establishes a threshold of approximately 3.75 V.

Under normal operating conditions, the output voltage is near 8 V, and the inverting input is well above the 3.75 V threshold. In this state, the comparator output remains low, pulled down via its open-drain output stage. If the 8 V output drops significantly — for example, due to a short circuit, overload, or MOSFET limiting action — the inverting input falls below the reference voltage. This causes the comparator to release its output, which is then pulled high via a 10 kΩ resistor. This rising edge is detected by the microcontroller as a fault condition.

The comparator output (8V_MONITOR) may be polled in firmware or used as an interrupt source. It provides a simple yet effective digital signal to indicate output health without the need for analog-to-digital conversion. Combined with the upstream current-limiting circuit, this monitor provides robust protection against user wiring errors and short-circuit events.

# Communications SUBSYSTEM

## CANBUS Interface

The MDD400 connects to the NMEA 2000 network via a standard 5-pin DeviceNet A-coded male connector. The CAN interface is isolated and filtered with ESD and surge protection devices to ensure compliance with marine EMC standards. A SN65HVD234DR[12] CAN transceiver interfaces the differential CANBUS signal to the ESP32's TWAI® pins. The SN65HVD234DR is a 3.3 V high-speed CAN transceiver rated for data rates up to 1 Mbps. It incorporates several features that improve bus reliability and robustness in harsh environments, including slope control for reduced EMI, a dominant state timeout to prevent bus lockup, and a low-power standby mode. The device is fully compliant with ISO 11898-2 [19] and is well suited for use in marine and automotive environments where noise immunity is critical.

### CANBUS Signal Conditioning

The CAN_H and CAN_L signals pass through:

15 pF ceramic capacitors to local ground (NET-C);

a 100 pF differential capacitor across the lines;

a common-mode choke (Murata ACT45B-510-2P-TL003 [23]);

an NUP2105LT1G dual TVS array [24];

and an SN65HVD234DR CAN transceiver [12].

These components form a compact, low-loss CAN filter network designed in accordance with Texas Instruments application notes [25][26]. The series capacitors (C23, C24, C18, C19) provide common-mode noise shunting and ESD suppression, while the 100 pF differential capacitor (C29) improves high-frequency noise immunity between CAN_H and CAN_L. The ACT45B common-mode choke attenuates high-frequency interference without significantly degrading the differential signal.

The short, direct PCB trace routing between the connector, filter components, and transceiver minimises parasitic inductance and avoids unnecessary layer transitions. The CANH and CANL signals are routed as a tightly coupled differential pair, preserving signal symmetry and maintaining consistent impedance. These layout strategies reduce radiated and conducted emissions and help ensure compliance with CISPR 25 [17] and ISO 11452-2 [18] in marine and automotive installations.

## Power Supply

The SN65HVD234 transceiver is powered from the 3.3 V digital supply rail shared with the ESP32. A three-stage bypass network is implemented at the VCC pin to ensure power integrity across a wide frequency range, consistent with the datasheet:

a 10 µF ceramic capacitor provides low-frequency bulk decoupling and stabilizes the supply during slower load changes;

a 100 nF ceramic capacitor is placed close to the VCC pin for general high-frequency bypassing; and

a 100 pF ceramic capacitor suppresses high frequency noise, e.g. fast transients caused by CAN signal switching.

### TTL I/O

The SN65HVD234 transceiver is powered from the 3.3 V digital supply rail shared with the ESP32. Logic-level CAN communication is implemented through the TX and RX pins:

[CAN_TX] connects directly to the transceiver’s D pin. A 10 kΩ pull-up resistor (R26) is used to ensure a defined idle state and limit noise susceptibility.

[CAN_RX] is connected to the transceiver’s R pin via a 68 Ω series resistor (R25), which limits inrush current, dampens reflections, and protects the ESP32 input from voltage spikes or overshoot.

Neither [CAN_TX] nor [CAN_RX] are strapping pins on the ESP32-S3, ensuring reliable bus behaviour during device flashing, reset or startup.

### Slope Control

The SN65HVD234 transceiver features a slope control mechanism on pin 5 (Rs), allowing designers to reduce the signal edge rate to suppress EMI. The slew rate is controlled by an external resistor to ground, and in the MDD400 a 10 kΩ pull-down resistor (R27) is fitted, resulting in a slew rate of approximately 15 V/µs.

Although the NMEA 2000 protocol mandates a data rate of 250 kbps, this value does not require a proportional edge rate. In differential signalling, reliable communication depends not only on bit rate but also on timing margins, signal rise/fall symmetry, and the receiver's ability to tolerate slower transitions. A slew rate of 15 V/µs is sufficient to support 250 kbps signalling over short drop cables (e.g. 1 m), such as those used to connect the MDD400 to the backbone. In practice, the edge rate limits the effective signalling bandwidth—not the fundamental data rate—so this setting provides ample timing margin without compromising protocol compliance.

Using a 10 kΩ pull-down offers a well-balanced trade-off: it substantially reduces high-frequency emissions (which scale with dV/dt) while remaining compatible with the stub length and impedance environment of a typical NMEA 2000 network. It also aligns with guidance provided by Texas Instruments in [25], which highlights 10 kΩ as an effective value for slope control in industrial and automotive CAN installations. The use of a resistor instead of an Rs capacitor simplifies the PCB layout and guarantees stable performance over temperature and component tolerances.

### Signal Integrity Simulation

A signal integrity simulation was conducted, assuming that the MDD400 is intended to connect to the NMEA 2000 network using a standard 5-pin DeviceNet A-coded male connector, with power and CAN signals provided via a short (1 m) drop cable. The NMEA 2000 backbone is assumed to comply with the NMEA 2000 physical layer specification [20], including termination via 120 Ω resistors at the bus extremities.

[Insert Figure: Signal Integrity Simulation of CAN_H and CAN_L]

## Legacy Serial Interface

The MDD400 supports an optional legacy serial interface designed to receive and/or transmit data compatible with SeaTalk I (single-wire, 12 V open-drain) and NMEA 0183 (12 V single-ended, receive-only). Given the 12 V signaling levels and the lack of galvanic isolation in typical legacy marine installations, the interface was engineered to maintain signal integrity and electromagnetic compatibility (EMC) through robust input filtering, carefully managed ground domains, and multi-stage protection.

The legacy interface accepts a single bidirectional signal line (ST_SIG), which is filtered and protected before being interfaced with the internal logic domain. SeaTalk operation uses a half-duplex scheme, sharing the line for both transmission and reception. NMEA 0183 operation is receive-only.

### Interface Conditioning and Power

Incoming power and signal lines are routed through a multi-stage filter network:

A common-mode choke (SMCM7060-132T) suppresses high-frequency noise;

Shunt capacitors (100 pF) to both digital and chassis ground domains provide RF filtering;

A bi-directional TVS diode (P6SMB15CA) clamps voltage spikes up to ±15 V;

Additional series resistance and diode clamping stages limit current and prevent latch-up.

This configuration offers substantial protection against ESD, conducted transients, and radiated EMI, satisfying CE and EN 55035 immunity requirements for the intended use case.

Internal signal buffering is provided by a two-stage transistor-based receiver. This stage isolates the MCU RX pin from the external signal domain and incorporates a voltage divider to safely shift the 12 V logic signal down to 3.3 V logic levels. The receiver is continuously active and handles both NMEA 0183 and SeaTalk receive functions.

Transmission is handled by a discrete open-collector driver controlled by the MCU TX pin and gated by a dedicated ST_EN signal. This stage mimics the behavior of legacy SeaTalk talkers by pulling the line low during a logic '0' and tri-stating otherwise. Series resistors and RC gate control limit EMI during transitions. An RC gate filter provides slew-rate limiting without affecting low-speed signaling (SeaTalk operates at 4800 bps).

All circuits are powered from the regulated 5 V domain (VCC), which is isolated from the external 12 V input by a pi filter and diode clamp. The entire legacy port shares a distinct ground domain (NET-C), which is joined to the main logic ground (GNDREF) via a common-mode inductor and local filtering.

If pre-compliance EMC testing indicates the SeaTalk interface as a source of excessive emissions or susceptibility, an alternative implementation using opto-isolators may be considered. This would allow complete galvanic isolation between the SeaTalk signal and the MDD400 logic domain, simplifying signal reference handling and improving immunity to ground loops or cable-borne transients. This approach would require powering the opto-isolated input side from the SeaTalk 12 V line or a derived source, and is best suited to receive-only applications such as NMEA 0183 listener modes.

### Circuit Description

#### Receiver

The input signal from the external ST_SIG line passes through diode D6A (1N4148) for reverse polarity protection and current limiting via R51. The signal is then applied to a pair of transistors (Q5A and Q5B) forming a differential receiver. These are biased by R47, R48, and R49 to provide signal translation and buffering.

The output of this stage is fed to [U1_RX], as 3.3 V UART-level serial data. This path is shared between NMEA 0183 and SeaTalk I operation. In both modes, the receiver operates continuously to capture serial input.

#### Transmitter

For SeaTalk I, which uses a single bidirectional wire, the MDD400 must also drive the ST_SIG line during transmission.

The transmit side employs Q6 (BC817) and Q8A (IMZ1A) as open-collector drivers, selectively enabled by Q7B and Q7A under control of the ST_EN signal. This allows the MDD400 to pull the signal line low while maintaining high impedance when not transmitting. A passive pull-up (R64) to +12 V ensures correct idle line voltage in the absence of an active transmitter.

[U1_TX] drives the signal logic, with R50 and R55 providing base current limiting and edge shaping.

### SeaTalk I Operation (Single Wire, RX/TX)

SeaTalk I is a single-wire bus using 12 V signaling, where idle = 12 V, logic 0 = pulled to 0 V. It requires careful coordination between transmit and receive functions. The MDD400 handles this using a half-duplex scheme:

when not transmitting, the ST_TX line is tri-stated, and the receiver monitors incoming traffic on ST_SIG;

to transmit, ST_EN is asserted to activate the driver circuitry, pulling ST_SIG low as needed;

an MCU-controlled TX driver sinks current during transmission and releases the line for receive or idle;

after transmission, the output drivers are immediately disabled to avoid contention with other devices on the bus; and

The RX stage remains active continuously and is tolerant of slow signal edges typical of long cable runs.

Timing and contention avoidance are handled in firmware. The circuit allows for reliable operation, even on long or noisy SeaTalk I networks.

### 3.2.4 NMEA 0183 Operation (RX Only)

For NMEA 0183 sources, the MDD400 operates in receive-only mode. The external talker is typically a GPS, depth sounder, or wind instrument. Although the NMEA standard allows for differential signaling (RS-422), most talkers in the marine environment operate in single-ended mode, and are electrically compatible with this circuit. The circuit design is also compatible with RS-422 (differential) data.

Only the receiver portion of the interface is used in this mode. The isolated, level-shifted signal is passed to [U1_RX]. No transmission is attempted, and the ST_EN signal remains low to ensure the transmitter remains disabled.

This configuration ensures the MDD400 is compatible with any standard NMEA 0183 talker without introducing noise or signal conflicts.

### Physical Connector

The physical interface for legacy serial signals is provided via a 3-pin header:

Pin 1 (RED): 12 V (power)

Pin 2 (BLACK): GND

Pin 3 (YELLOW): SIG (SeaTalk I / NMEA 0183 signal line)

This layout allows for simple three-wire connectivity using standard SeaTalk I connector.

#### Power Isolation

In production, the SeaTalk +12 V pin is intentionally left unconnected to the MDD400 internal power supply. This avoids introducing a second power source and the associated risks of conflicting voltage domains or ground loops. As a result, the MDD400 must be powered exclusively from the NMEA 2000 backbone, and any SeaTalk devices must receive power independently from the SeaTalk bus.

This isolation is implemented by omitting the polarity protection diode on the SeaTalk power input. For advanced configurations, populating this diode enables power input from the SeaTalk bus. Additionally, a normally open solder jumper labeled ST_12V is provided to allow bridging the SeaTalk and NMEA 2000 power rails. This option is intended for edge cases, such as powering a single connected SeaTalk device that is not attached to a live SeaTalk bus. Use of this jumper should be avoided in typical installations to maintain electrical isolation and CE-compliant power domain separation.

#### EMC, ESD and Ground

The legacy port is exposed via a 3-pin header (12 V, GND, SIG). In this implementation, the +12 V line is left unconnected, and the interface is powered entirely from the internal 5 V rail. The GND pin (SeaTalk shield) is connected to GNDREF through a 1 µH inductor, which provides a low-impedance signal reference path while attenuating high-frequency noise.

Although the original SeaTalk protocol uses GND as a signal return and implicit shield, this arrangement preserves CE compliance by preventing direct coupling of noisy external grounds into the device ground plane. The use of a separate shield pin was considered, but real-world installations route signal returns through the GND wire, making this unnecessary.

The input filter and grounding strategy ensure that EMC performance meets CE and FCC Class B requirements for conducted and radiated emissions. Immunity to electrostatic discharge, EFT, and surge is provided by a combination of TVS clamping, series impedance, and low-inductance layout.

In total, there are six possible wiring permutations of the three legacy interface pins (12V, GND, SIG). These are evaluated in the Table 4:

Table 4: Legacy Connector Wiring Permutations

| Wiring Order | ST_SIG | 12V_IN | GND | Outcome | Protected? |
| --- | --- | --- | --- | --- | --- |
| SIG, 12V, GND | ST_SIG | 12 V | GND | ✅ Correct connection — normal operation | ✅ |
| 12V, SIG, GND | 12 V | ST_SIG | GND | ST_SIG pulled high — D6A blocks current, input already expects 12 V | ✅ |
| SIG, GND, 12V | ST_SIG | GND | 12 V | GND applied to 12V_IN — D8 blocks reverse current | ✅ |
| GND, 12V, SIG | GND | 12 V | ST_SIG | GND on ST_SIG — valid electrical state (LOW), no damage | ✅ |
| 12V, GND, SIG | 12 V | GND | ST_SIG | Same as #2 — signal input sees 12 V, safely blocked by D6A | ✅ |
| GND, SIG, 12V | GND | ST_SIG | 12 V | GND to ST_SIG — logic LOW input, 12 V on GND blocked by D8 | ✅ |

All six possible wiring permutations are electrically safe. There are no failure modes resulting from any pin swap of the 3-pin legacy connector. While incorrect wiring may prevent the interface from functioning (e.g., no data received), no permanent damage will occur to the MDD400 or connected devices. This level of fault tolerance is particularly valuable for field installations and testing scenarios where connector orientation or labeling may not always be foolproof.

# Sensors

## Power Sensor

The MDD400 continuously monitors its own supply voltage and current draw to provide diagnostic data, support fault detection, and enable intelligent power management. These measurements are digitized via the ESP32-S3’s internal ADCs and are sampled regularly by firmware. This section describes the hardware circuits used for voltage and current sensing.

### Voltage Sensing

The input voltage is sensed downstream of the active protection circuit, at the output of Q13 (labelled V_PROTECTED), and represents the voltage seen by all internal circuitry. A passive voltage divider consisting of R52 (22 kΩ) and R57 (4.7 kΩ) scales the protected voltage to within the 0–3.3 V input range of the ESP32-S3 ADC. This scaled voltage is then buffered by an op-amp (U8A), configured as a unity-gain follower using a TLV9002IDR. The buffer ensures a low-impedance, noise-free signal suitable for accurate ADC conversion and is connected to [VSS_ADC], which is configured as an ADC input.

The voltage divider ratio is ​≈0.176. Given a 3.3 V ADC input range, the corresponding maximum measurable input voltage is ​≈18.75 V.

With a 12-bit ADC resolution, the measured voltage can be calculated by multiplying the ADC reading by a factor of 0.00458.

### Current Sensing

The input current is measured using a low-side 100 mΩ shunt resistor placed in the return path of the main input filter stage. The voltage developed across this resistor is proportional to the load current and is used as the input to a non-inverting amplifier built around a TLV9002IDR op-amp (U8B).

The amplifier uses a 10 kΩ input resistor (R56), a 150 kΩ feedback resistor (R54), and a 3.3 kΩ resistor to ground (R58), producing a gain of approximately 47. With the 100 mΩ shunt resistor the current measurement range is 0 – 715mA: double the normal operating peak current of the MDD400, but below the upper limit of the protection circuit. This ensures that any overcurrent condition results in a distinct and easily detectable ADC output, simplifying firmware-based fault detection.

The amplified signal is routed to (CURRENT_ADC) for sampling by the ESP32-S3’s ADC.

This design allows accurate current measurement without introducing significant voltage drop or power loss in the ground return path. The amplified signal is suitable for monitoring both steady-state load and transient inrush currents.

With a 12-bit ADC resolution, the measured current can be calculated by multiplying the ADC reading by a factor of 0.0001734.

## Temperature Sensor

Temperature monitoring is a key diagnostic feature of the MDD400, providing insights into internal thermal conditions that may affect reliability, performance, and user experience. The design includes two complementary temperature sensors: the internal ESP32-S3 junction sensor and an external high-accuracy I²C temperature sensor (TMP112) mounted near the display.

Together, the ESP32-S3’s internal sensor and the external TMP112 provide robust and layered visibility into the thermal state of the MDD400, improving resilience and field reliability.

### Internal Microcontroller Temperature Sensor

The ESP32-S3 microcontroller includes an integrated temperature sensor, which is sampled periodically by firmware. This sensor provides an estimate of the chip’s internal junction temperature and is useful for monitoring system-level heating caused by sustained processor load, ambient temperature rise, or enclosure heat buildup.

While not calibrated for precision use, this internal sensor allows early detection of abnormal heating, and supports thermal diagnostics across a typical range of –20 °C to +125 °C. Measured values are exposed via the system’s diagnostics interface and may be used in conjunction with voltage and current data to identify thermal trends.

However, due to its location within the silicon die, this sensor does not reflect the temperature of nearby components — particularly those affected by external heat sources such as direct sunlight.

### External Display Temperature Sensor

To provide more meaningful thermal data near the display panel, a dedicated TMP112AIDRLR temperature sensor is mounted on the main board, in close proximity to the rear of the capacitive touch LCD. This sensor is connected to the ESP32-S3 via the I²C bus pins [SCL] and [SDA]. The TMP112 device address is configured via the ADD0 pin to 0x48, the default address when ADD0 is tied to GND.

The TMP112 supports 12-bit resolution (0.0625 °C per bit) with typical accuracy of ±0.5 °C across the range –40 °C to +125 °C. It is powered from the regulated logic supply (VCC), and includes 100 pF and 1 µF local bypass capacitors (C55 and C56) for stability. A 10 kΩ pull-up resistor (R38) is fitted on the ALERT line, allowing firmware to configure an interrupt for temperature threshold crossing if needed.

This sensor provides a reliable indicator of display-local temperature — particularly important in marine environments where direct sun exposure can rapidly elevate surface temperatures beyond the LCD’s rated operating range. The DWIN DMG48480F040_01WTC display does not incorporate an internal temperature sensor, so external monitoring is required. When high temperature is detected, firmware may respond by dimming the backlight, issuing a warning, logging a diagnostic event or turning off the display by pulling [DISP_EN] low.

## Ambient Light Sensor

The MDD400 incorporates an ambient light sensor to enable automatic backlight brightness control and improve visibility in changing lighting conditions. This functionality is particularly valuable in marine environments, where ambient light can vary widely—from direct sunlight to nighttime operation. Automatic adjustment reduces power consumption, minimizes glare, and extends the useful lifetime of the display.

### OPT3004 Digital Ambient Light Sensor

The system uses the OPT3004DNPR, a low-power digital ambient light sensor with high dynamic range and spectral response closely matching the human eye. The sensor communicates with the ESP32-S3 via I²C and is connected to the same bus as the TMP112 temperature sensor ([SCL] and [SDA] pins). The OPT3004 address is configured via the ADDR pin (connected to GND), setting its I²C address to 0x44.

The OPT3004 provides a 16-bit lux measurement over a dynamic range of 0.01 lux to 83,000 lux, automatically scaling via its built-in exponent/mantissa format. This range is ideal for both indoor/dim environments and direct sunlight detection. The sensor includes an interrupt output (INT), which is not used in the current design but is brought out to a test point for future expansion.

The device is powered from the 3.3 V logic supply (VCC) and is locally decoupled with 100 pF and 1 µF capacitors (C7 and C8). A 10 kΩ pull-up resistor (R9) is used on the I²C lines. The sensor is positioned close to the front panel of the device to ensure accurate readings of incident light on the display surface.

### Firmware Use

The ESP32-S3 periodically polls the OPT3004 and uses the reported lux value to adjust the backlight brightness of the LCD panel. The lux value is scaled to a target brightness level using a configurable mapping curve, which may include hysteresis or smoothing to avoid flicker during rapid ambient light transitions.

In low-light conditions, the firmware reduces the backlight intensity to minimize power consumption and glare. In bright conditions, full brightness is applied to maintain readability. The ambient light value is also recorded in system diagnostics and may be used to correlate with temperature trends or display behavior.

The use of a dedicated ambient light sensor improves user experience and provides adaptive behavior in real-world conditions where lighting varies significantly throughout the day.

## Wind Transducer Interface

This analog interface was designed to maximize compatibility with legacy analog wind transducers that output analog angle and speed pulse signals.

The architecture leverages single-supply op-amps and logic-level conditioning to integrate cleanly with the ESP32-S3’s internal peripherals. Together with a regulated 8 V analog supply (Section 2.2.3), the MDD400 supports legacy transducers with minimal software overhead and high confidence in signal fidelity.

### Circuit DESIGN

#### Transducer Power Supply

The transducer is powered from a regulated 8 V rail as described in Section 2.2.3. Combined, the entire wind transducer system is well within the 100 mA output rating of the 8 V /100 mA regulated power supplied by this design.

#### Electromagnetic Interference Suppression

All transducer inputs include LC filtering (10 µH series inductors with 47 nF or 100 nF capacitors) to attenuate RF interference, with particular care given to the high-impedance analog X/Y signals. Analog ground routing and component layout were optimized to minimize loop area and ensure stability under variable load conditions on the 8 V rail.

#### Wind Angle Signal Conditioning

The direction sensing circuit includes two independent analog channels that receive and condition voltage signals representing wind direction:

Two analog wind angle signals from the transducer are routed through LC low-pass filters (L5/C11 and L1/C1, respectively) to suppress high-frequency interference. Each signal then passes through a resistive divider (e.g. R21/R22 for X) and a DC blocking capacitor (C21/C2), removing the transducer's DC bias and allowing the signal to be re-biased around a local reference.

Each AC-coupled signal is biased to mid-rail via a resistor divider (R23/R24 or R15/R14) that establishes a 2.5 V reference from the VCC rail (typically 3.3 V). The resulting centered signals are fed into single-supply rail-to-rail operational amplifiers (TLV9002) configured as non-inverting buffers. These op-amps provide low-impedance drive and isolation for the downstream analog-to-digital converters.

The processed outputs are connected to dedicated ADC-capable [WIND_X_ADC] and [WIND_Y_ADC]. With appropriate sampling and arctangent computation, these signals yield a continuous wind direction angle from 0° to 360°.

#### Wind Speed Signal Conditioning

The supported transducers generate a pulse train on the wind speed signal line via an opto-interrupter, hall-effect sensor or open-collector transistor. This signal typically pulls the speed line to 0 V on every pulse and switches state twice per anemometer cup rotation.

The anemometer output (WIND_SPD) is a passive open-collector transistor pulsed to ground inside the transducer. This signal is pulled up to the 8 V supply via R5 and further attenuated by R6/R8 before reaching the comparator input.

The comparator (LM393) converts the analog pulse into a 3.3 V logic-compatible TTL signal. The reference voltage for comparison is derived via a simple resistive divider (R4/R7) from the 8 V rail and decoupled by C3. The hysteresis resistor (R9) introduces positive feedback to improve noise immunity and edge clarity.

The TTL output is connected to the [WIND_SPEED_TTL] pin, configured as an interrupt-capable edge detector. Each falling edge represents a known angular displacement of the anemometer, allowing firmware to compute wind speed as a function of pulse frequency. Calibration to wind speed is performed by the firmware and is user configurable.

This circuit is designed to work reliably with both Raymarine and B&G anemometers, as both use fundamentally similar open-collector pulse generation mechanisms.

### Compatible Wind Transducers

The wind transducer interface is compatible with a wide range of legacy masthead wind transducers, including popular masthead transducers from Raymarine /Autohelm and Navico (B&G, Simrad) that output analog wind direction and pulse-based wind speed signals. While they differ in how they encode wind angle, these transducers provide a similar open-collector pulse signal for wind speed.

We studied several generations of these transducers and found them to present very similar electrical interfaces. The shared signal conditioning circuitry enables clean digital interfacing with the ESP32-S3 MCU.

The interface circuit is NOT compatible with Raymarine RotaVecta™ transducers.

#### Raymarine ST60-style Masthead Wind Transducer

For wind speed, the Raymarine transducer provides a pulse signal (Yellow wire) generated by an opto-interrupter. This open-collector signal is fully compatible with the WIND_SPEED comparator circuit, requiring no modification.

For wind angle, the Raymarine transducer outputs analog sine and cosine signals representing wind angle. These form a resolver-like system. The two channels (Blue and Green wires) vary in voltage as the vane rotates and are biased around half the transducer’s supply voltage (~4 V on an 8 V rail). The signals are processed through the shared analog conditioning circuit and sampled by the ESP32-S3.

In firmware, wind angle is derived using:

θ=atan2(Y,X)

We briefly discuss the circuit design of the later Raymarine E22078/E22079, noting that the earlier ST60/50 and Autohelm transducers are electrically and pin compatible.

The Raymarine  E22078/9  uses a Melexis Sentron 2SA-10 dual-axis Hall-effect sensor, which draws a steady supply current of approximately 16 mA as per its datasheet [26]. In addition, the transducer includes two TL914 dual op-amps that buffer the X/Y vane signals and provide local power conditioning for the sensor. While these op-amps and associated passive components contribute additional quiescent current, the total consumption of the wind transducer is estimated to remain below 25 mA in normal operation. The anemometer circuit, which uses an opto-interrupter with a series 560 Ω resistor, contributes transient pulses but negligible average current. Combined, the entire wind transducer system is well within the 100 mA output rating of the 8 V regulated power supply.

Earlier ST60/50 and Autohelm transducers are electrically and pin compatible with the later E22078/9. They are much simpler devices, employing Hall-effect sensors from Honeywell for both the sine/cosine and speed sensors. These early transducers draw less power than the E22078/E22079 models.

#### B&G Wind Transducers

The B&G masthead transducer uses three phase-shifted analog outputs (Red, Green, Blue), spaced 120° apart. These form a synchro-like system. The MDD400 requires only two of these signals to compute angle and typically uses the Red and Blue wires, connected to the WIND_X and WIND_Y channels. These pass through the same analog signal chain as described above.

In firmware, a modified trigonometric transform is used to calculate wind direction:

θ=atan2(VB,VG)+offset

Where VB​ and VG​ are the voltages sampled from the Blue and Green channels respectively, and the offset compensates for phase alignment.

The B&G anemometer uses a similar open-collector switching circuit to Raymarine for wind speed, producing a 2-pulse-per-rotation open-collector output. This signal is fully compatible with the comparator-based speed sensing circuit and requires no additional conditioning.

The B&G 213 transducer operates nominally at 6.5 V and has been verified to tolerate input voltages up to 8 V. Diagnostic documentation indicates the use of analog vane signals with phase-shifted sinusoidal outputs (Red, Green, Blue) and a pulse-type wind speed output on the Violet line. The angle transducers employ magnetic coupling of the wind vane shaft to an inductive antenna.

Under normal operation, the B&G 213 unit draws an estimated total current of 25 to 30 mA, consistent with its analog signal architecture and passive sensing elements. Compatibility testing confirmed stable operation of the unit at 8.0 V, with no signal distortion or thermal issues observed. —

Since the MDD400 uses a Raymarine-compatible input layout, wiring B&G transducers requires matching B&G colors to expected X/Y/Speed inputs. Only two of the three angle phase outputs (typically GREEN and BLUE) are required to compute wind angle via firmware.

Table 3: B&G Wind Transducer Wiring Matrix (post March 1996 devices)

| B&G Wire | Function | Connect to MDD400 Pin | MDD400 Label | Signal Path |
| --- | --- | --- | --- | --- |
| Red | Wind angle (Phase R) | — | — | — |
| Green | Wind angle (Phase G) | GREEN | Y | → WIND_Y (ADC) |
| Blue | Wind angle (Phase B) | BLUE | X | → WIND_X (ADC) |
| Violet | Wind speed pulse | YELLOW | SPD | → WIND_SPEED_TTL |
| Black | 0V / Ground | SHIELD | GND | GNDREF |
| Orange | +6.5V Supply voltage | RED | +8V | → 8 V LDO Regulator |

Only two of the three angle outputs are required. Raymarine-compatible wiring expects BLUE = X and GREEN = Y, which matches the most straightforward firmware mapping for B&G systems. B&G transducer models manufactured pre-March 1996 present a variety of colour – signal configurations. Please refer to the B&G service documentation[28] when connecting these older transducers.

The transducer interface presents a high impedance on all signal lines, while the 8 V power line is polarity and short circuit protected. Incorrect wiring will result in no

Special Considerations:

The B&G units were originally powered from a nominal 6.5 V rail but have been verified to operate reliably at 8 V with current draw well under 100 mA.

Ensure polarity is observed when connecting legacy B&G cables to the MDD400 breakout — especially for older installations with undocumented color assignments.

If needed, an inline adapter cable can be fabricated to convert between B&G 7-pin or 8-pin DIN connectors and the MDD400 connector block.

This matrix allows technicians or DIY users to connect legacy B&G transducers to the MDD400 without modification, ensuring backward compatibility across a wide range of vessels.

# USER Interface

## Overview and HMI Elements

The MDD400 features a minimalist yet powerful human-machine interface designed for intuitive operation in marine and RV environments. The interface comprises a high-resolution capacitive touch display as the sole means of user input and output. There are no physical buttons or rotary controls, ensuring a sealed, low-maintenance front panel with IP-rated ingress protection.

User interaction is structured around a simple swipe and tap paradigm:

Swipe left/right: Navigate between data pages.

Swipe down: Access the control panel for brightness, data source selection, and other system-wide settings.

Swipe up: Reveal per-page options and configuration overlays.

Tap on-screen labels: Toggle display modes such as AWS/TWS, magnetic/true heading, or knots/Beaufort.

This interface prioritizes clarity, ease of use, and visibility under all lighting conditions, with on-screen brightness control and automatic timeout for the touch layer after inactivity.

## Power Supply

The display module is powered from the regulated 5 V supply rail, controlled by a dedicated power switch circuit to allow selective shutdown. This circuit uses a P-channel MOSFET (AO3407) gated by a BJT (S8050) controlled via the [DISP_EN] pin. When [DISP_EN] is driven high, the P-MOSFET turns on, supplying 5 V (VSS) to the display module.

Decoupling is provided by a 100 µF/50 V and a 10 µF/25 V capacitor in parallel near the display connector (C25 and C21). This provides adequate bulk capacitance and transient suppression for the display's inrush and dynamic load.

The display consumes up to 245 mA at 5 V when the backlight is fully illuminated, and as little as 75 mA with the backlight off, as verified in Section 2.4 of the design report and the DWIN datasheet.

## Touch Display

### Characteristics

The MDD400 uses a DWIN DMG48480F040_01WTC capacitive LCD module. Key characteristics include:

Panel type: 4.0" IPS TFT with 262K colors and 480×480 resolution.

Viewing angle: 85° in all directions.

Brightness: 250 nits (typical) with PWM dimming control from 0–100%.

Backlight control: Brightness is dynamically adjusted based on ambient light conditions using the MDD400’s onboard light sensor.

Touch: 1-point capacitive touch (G+G structure), supporting continuous sliding gestures.

Backlight life: >10,000 hours at full brightness.

Touch life: >1,000,000 touches with 6H surface hardness.

Power consumption: <2 W maximum.

### Communication

The display communicates over UART2, using TTL signal levels:

TX (MCU → Display): [U2_TX] pin; and

RX (Display → MCU): [U2_RX] pin.

The interface operates using the DGUS II protocol, which abstracts screen logic into memory pages, variables, and commands. This approach minimizes MCU load and ensures responsive screen updates even on lower-speed links.

Although the display supports a wide range of baud rates (up to 3.2 Mbps), initial firmware defaulted to 115200 bps for compatibility. However, testing confirmed that reliable, error-free communication is maintained at 460800 bps, even in electrically noisy environments. This baud rate is now used in production, significantly improving screen responsiveness during page transitions and configuration updates.

The interface uses the standard 8N1 format (8 data bits, no parity, 1 stop bit), consistent with DGUS II system defaults.

Practical testing confirms that the DWIN DMG48480F040_01WTC display can reliably sustain sprite and GUI element updates at 30 frames per second when communicating at 460800 bps via UART2. This performance is enabled by the DGUS II protocol’s use of preloaded assets and indexed object control, rather than full bitmap transmission. While large image transfers are constrained by UART bandwidth, command-level updates—such as repositioning, toggling, or redrawing preloaded objects—exhibit no perceptible lag at 30 FPS under typical usage scenarios.

## Touch Screen Operation

The MDD400 uses a capacitive touch screen that supports single-point gestures such as swipes and taps. Touch inputs are processed by the internal controller of the DWIN display, using mutual capacitance sensing to detect changes in the local electrostatic field when a conductive object, such as a human finger, approaches the panel surface.

### Timeout

To conserve power and improve durability, the touch interface automatically disables after a configurable period of user inactivity. The default timeout is 15 s, after which the touchscreen is disabled until the next touch. This timeout can be adjusted in firmware.

### Wet-Weather Usability

Capacitive touch screens are typically vulnerable to false inputs when wet, as water droplets can interfere with the electrostatic field used to detect touch. Early MDD400 prototypes mitigated this by requiring a physical button press to activate the touchscreen. However, real-world testing during long-term marine deployment showed this precaution to be unnecessary. In testing, the touch interface remains fully usable in wet conditions, including during rain or spray events. This robustness is attributed to several factors:

the display uses mutual-capacitance sensing, which is less susceptible to false inputs from stray conductive paths such as water films;

the housing is constructed from non-conductive material and affixed using 3M 300LSE adhesive, providing full electrical isolation from the vessel;

the housing is mounted to a GRP hull, offering no low-impedance return path to earth; and

users are generally electrically isolated (e.g., by shoes or non-conductive flooring), reducing the coupling between finger, water, and system ground.

Together, these factors form a floating capacitive system with limited return paths, reducing the influence of water and parasitic capacitance.

### Grounding Considerations on Metal-Hulled Vessels

On metal vessels, the hull and structure may provide a conductive path to earth. If the display enclosure is bonded—intentionally or inadvertently—to a grounded surface, the capacitive behaviour of the touch interface may change. In particular:

water films may couple more effectively to system ground, increasing the likelihood of false inputs; and

users in contact with grounded metal may present a stronger capacitive target, potentially altering sensitivity.

To maintain performance in these installations, the following measures are recommended:

ensure mechanical isolation between the enclosure and any conductive mounting surfaces using non-conductive spacers or gaskets;

avoid bonding the display ground to hull or battery ground unless specifically required by EMC or safety considerations; and

maintain the current floating configuration of the touchscreen ground unless testing indicates a need for tighter control.

### Further Testing

Additional testing is planned to verify touchscreen reliability on metal-hulled vessels and in conductive mounting environments. Recommended tests include:

operating the touchscreen under simulated rain or spray conditions while bonded to a grounded metal plate;

comparing touch sensitivity and false input rate with the user isolated vs. grounded; and

verifying whether localized grounding (e.g. display GND tied to chassis) affects performance in wet conditions.

These tests will guide design guidelines for future installation scenarios and ensure consistent behavior across vessel types.

## Audio Alerts

A magnetic buzzer is driven by a PWM input on the [AUDIO_PWM] pin, allowing the firmware to combine tones from the display and system alerts.

The MDD400 display also includes an integrated audio output signal (labelled DISP_AUDIO) that is routed to the system buzzer/speaker driver.

The two audio signals are buffered by a BJT (Q1) and used to drive a P-MOSFET (Q2), which controls current through the magnetic buzzer (BZ1). An R-C filter (18 Ω / 470nF) creates a high pass filter with a cutoff frequency of 8.8kHz. This smooths the tone, slightly reduces volume, and rolls off the high-frequency buzz, giving a rounder, more pleasant beep, especially at ~2–3 kHz.

This circuit supports:

system error or warning tones triggered by firmware;

acknowledgement or status tones from the DGUS display (e.g., on touch events); and

PWM-controlled modulation for generating tones of varying pitch and duration.

The buzzer (MLT-8530) is a surface-mount transducer capable of >80 dB sound pressure at 5 V. Its compact size and efficient drive circuit allow it to operate reliably across the full temperature and humidity range specified for the device.

# Expansion Connectors

Expansion port connectors are not populated on consumer versions of the MDD400. Only the MDD400-DEV version gives makers and the open-source community access these connectors.

These connectors expose the microcontroller pins and no protection is provided to ensure the maximum utility. The [SDA], [SCL], [EN] and [BOOT] pins are pulled up to Vcc by 10k Ω resistors. All other pins are not pulled up or down to maximise utility.

## ESP-PROG

Describe the ESP-PROG here

### Flash Programmer

Used in production for flashing firmware

Is a through-hole part. In production a dedicated programmer with pogo pins is used to interface to the same footprint

### JTAG Debugging Port

The JTAG connector (1.27mm pitch, 2x5 pins) is compatible with the ESP-PROG cable.

It is only populated tin the MDD400-DEV version.

The connector can be used for expanding four MCU pins to off-PCB functions. The connector provides GND and 5v. In addition, 3.3v can also be provided (on pin 10 of the connector) by closing the adjacent solder jumper.

## I2C

Has a 4 pin, 2.54mm pitch male header, compatible with DuPont cable/connectors. Exposes GND, 3v3, [SDA] and [SCL]

## One-Wire

The [1-WIRE] pin, GND and 5V is connected to this 3-pin, 2.54mm pitch male header compatible with DuPont cable/connectors. Provides an interface for industry standard DS18B20 one-wire temperature sensors.

# Processor

## Description

## flash storage partitioning

## GPIO Mapping

## WiFi

## Bluetooth/BLE

# PCB Layout and Thermal Design

## Board Stackup

## Earth Planes

## Functional Layout

## Thermal Design

## Mechanical

### Connectors

### Dimensions

### Fasteners

### Fabrication strategy

# 7. Validation and Test Plan

appendix with the full compliance matrix or EMC test plan in the next sections.

# 8. Bill of Materials Summary

# 9. Conclusion and Future Work

# References

[1] ISO 7637-2:2011, “Road vehicles — Electrical disturbances from conduction and coupling — Part 2: Electrical transient conduction along supply lines only,” International Organization for Standardization, 2011.

[2] IEC 61000-4-2:2008, “Electromagnetic compatibility (EMC) — Part 4-2: Testing and measurement techniques — Electrostatic discharge immunity test,” International Electrotechnical Commission, 2008.

[3] ON Semiconductor, “SS34 — Surface Mount Schottky Power Rectifier,” Rev. 7, Nov. 2017. [Online]. Available: https://www.onsemi.com/pdf/datasheet/ss34-d.pdf

[4] Texas Instruments, “Load Dump Protection Fundamentals,” SNVA681A, May 2019. [Online]. Available: https://www.ti.com/lit/an/snva681a/snva681a.pdf

[5] Monolithic Power Systems, “From Cold Crank to Load Dump: A Primer on Automotive Transients,” [Online]. Available: https://www.monolithicpower.com/en/blog/post/from-cold-crank-to-load-dump-a-primer-on-automotive-transients

[6] Texas Instruments, “TPS54560B-Q1 — 60-V, 5-A, Wide Input, Step-Down DC/DC Converter,” Rev. E, Jan. 2021. [Online]. Available: https://www.ti.com/lit/ds/symlink/tps54560b-q1.pdf

[7] Texas Instruments, “LM1117 — 800-mA Low-Dropout Linear Regulator,” Rev. T, Feb. 2015. [Online]. Available: https://www.ti.com/lit/ds/symlink/lm1117.pdf

[8] Texas Instruments, “LP2951 — Micropower Voltage Regulator with Programmable Output,” Rev. T, Oct. 2016. [Online]. Available: https://www.ti.com/lit/ds/symlink/lp2951.pdf

[9] Texas Instruments, “TLV9002 — 1-MHz, Low-Noise, RRIO, General-Purpose Operational Amplifier,” Rev. F, Dec. 2022. [Online]. Available: https://www.ti.com/lit/ds/symlink/tlv9002.pdf

[10] Texas Instruments, “LM393 — Low-Power, Dual Voltage Comparator,” Rev. M, Aug. 2016. [Online]. Available: https://www.ti.com/lit/ds/symlink/lm393.pdf

[11] Texas Instruments, “OPT3004 — Ambient Light Sensor,” Rev. E, Oct. 2020. [Online]. Available: https://www.ti.com/lit/ds/symlink/opt3004.pdf

[12] Texas Instruments, “SN65HVD234 — 3.3-V CAN Transceiver with Standby Mode,” Rev. K, Mar. 2015. [Online]. Available: https://www.ti.com/lit/ds/symlink/sn65hvd234.pdf

[13] Texas Instruments, Protection against Unsuppressed Load Dump in Automotive Systems using LM74930-Q1, SNOAAA1, May 2022. [Online]. Available: https://www.ti.com/lit/ab/snoaaa1/snoaaa1.pdf

[14] Texas Instruments, Load Dump and Cranking Protection for Automotive Backlight LED Power Supply, SNVA681A, May 2019. [Online]. Available: https://www.ti.com/lit/an/snva681a/snva681a.pdf

[15] DWIN Technology, DMG48480F040_01WTC – 4.0" Intelligent Display, COF Series, [Online]. Available: https://www.dwin-global.com/4-0-inch-intelligent-display-model-dmg48480f040_02wtcz02cof-series-product/

[16] SMC Diode Solutions, SM8S20CA thru SM8S43CA – 6.6kW Surface Mount Transient Voltage Suppressors, Rev. N2149. [Online]. Available: https://www.smc-diodes.com/propdf/SM8S20CA%20THRU%20SM8S43CA%20N2149%20REV.-.pdf

[17] Espressif Systems, ESP32-S3-WROOM-1/1U/2 Datasheet: Wi-Fi & Bluetooth LE SoC Modules, v1.9, 2023. [Online]. Available: https://www.espressif.com/sites/default/files/documentation/esp32-s3-wroom-1_datasheet_en.pdf

[18] CISPR 25:2021, Vehicles, boats and internal combustion engines – Radio disturbance characteristics – Limits and methods of measurement for the protection of on-board receivers, International Electrotechnical Commission (IEC), 2021.

[19] ISO 11452-2:2021, Road vehicles — Component test methods for electrical disturbances from narrowband radiated electromagnetic energy — Part 2: Absorber-lined shielded enclosure, International Organization for Standardization, 2021.

[20] NMEA, NMEA 2000® Standard for Serial-Data Networking of Marine Electronic Devices, Version 3.200, National Marine Electronics Association, 2018.

[21] RVIA, Recommended Practice for RV-C: RV-C Protocol Definition, Recreational Vehicle Industry Association (RVIA), 2022. [Online]. Available: https://www.rvia.org

[22] Texas Instruments, LP2951 — Micropower Voltage Regulator with Programmable Output, Rev. T, Oct. 2016. [Online]. Available: https://www.ti.com/lit/ds/symlink/lp2951.pdf

[23] Murata, "ACT45B-510-2P-TL003 — Common Mode Choke for CAN Bus Applications," [Online]. Available: https://www.murata.com/en-us/products/productdata/8807038415390/QTN0099C.pdf

[24] ON Semiconductor, "NUP2105L — Dual Line ESD Protection Array," Rev. 5, Oct. 2019. [Online]. Available: https://www.onsemi.com/pdf/datasheet/nup2105l-d.pdf

[25] ON Semiconductor, "SMF15CA — 200 W Transient Voltage Suppressor," Rev. 2, Mar. 2017. [Online]. Available: https://www.onsemi.com/pdf/datasheet/smf15ca-d.pdf

[26] Melexis Technologies NV, Dual-Axis Magnetic Sensor IC - 2SA-10, Rev. 2.4, Melexis, Apr. 2010. [Online]. Available: https://gmw.com/wp-content/uploads/2019/02/2SA10.pdf

ISO7637 Test Pulses | Video | TI.com